Specification for MT – 241026/SLH/B

LHCP Embedded Antenna

865 - 956 MHz, 3dBic

THIS DOCUMENT AND THE INFORMATION CONTAINED IN IT ARE PROPRIETARY AND CONFIDENTIAL TO MTI.  NO PERSON IS ALLOWED TO COPY REPRINT REPRODUCE OR PUBLISH ANY PART OF THIS DOCUMENT NOR DISCLOSE ITS CONTENTS TO OTHERS NOR MAKE ANY USE OF IT NOR ALLOW OR ASSIST OTHERS TO MAKE ANY USE OF IT, UNLESS BY THE PRIOR WRITTEN EXPRESS AUTHORIZATION OF MTI AND THEN ONLY TO THE EXTENT AUTHORIZED.

# (C) COPYRIGHT – MTI WIRELESS EDGE LTD.

| REVISION RECORD |  |  |  |  |
| --- | --- | --- | --- | --- |
| DATE | APVD | DESCRIPTION | ECO | REV |
| 7.7.10 | Yael h. | RELAESED | DC51 | A |

| 56 | 55 | 54 | 53 | 52 | 51 | 50 | 49 | 48 | 47 | 46 | 45 | 44 | 43 | SHEET |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | REV |
| 42 | 41 | 40 | 39 | 38 | 37 | 36 | 35 | 34 | 33 | 32 | 31 | 30 | 29 | SHEET |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | REV |
| 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | SHEET |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | REV |
| 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | SHEET |
|  |  |  |  |  |  |  |  |  |  | A | A | A | A | REV |

| # PRODUCT STATUS: |  |  |  |
| --- | --- | --- | --- |
| # PRODUCTION | # PREPRODUCTION | # IN DEVELOPMENT | # PROPOSAL |

| CONTRACT NO |  |  |  |
| --- | --- | --- | --- |
| DATE | SIGN | NAME | ACTION |
| 7.6.10 | YAEL | YAEL HRASH | PREP |
| 7.7.10 | D.Z | DANIEL ZERAH | CHKD |
| 7.7.10 | Y.M | Y.MILLER | CHKD |
|  |  |  | CHKD |
| 23.6.10 | S.Z | SERGEY ZEMLIAKOV | DSGN |
| 7.6.10 | M.R | MOTI ROSTMI | PROJ |
|  |  |  | MFG |
| ##### 4.7.10 | ##### MOTI | ##### MOTI MOTOVICH | QA |
| 4.7.10 | R.D | REUVEN DRORI | MARK. |

##### SPECIFICATION

Flat Panel Embedded Antenna

| #### MTI PART NUMBER | ## MT – 241026/SLH/B |
| --- | --- |
| Regulatory Compliance | RoHS , CE 0682 |
| 1. ELECTRICAL |  |
| # Frequency Range | 865 - 956 MHz |
| Gain | 2.0 dBic (min) 3.5 dBic (max) |
| VSWR | 1.3 :1 (typ) , 1.5 : 1(max) |
| 3 dB Beamwidth Az El | 83° (typ) 83° (typ) |
| Polarization | LHCP |
| Axial Ratio @ Boresight | 2 dB (typ) , 3 dB (max) @ 865-870 MHz 1 dB (typ) , 2.5dB (max) @ 902-928 MHz 3 dB (typ) , 4 dB (max) @ 950-956 MHz |
| F / B Ratio | -10 dB (typ) |
| Input Impedance | 50 ohm |
| Input Power | 6W (max) |
| Lightning Protection | DC GROUNDED |
| 2. MECHANICAL |  |
| Dimensions(LxWxD) | 95 x 95 x15 * mm (max) |
| Weight | 220gr (max) |
| Connector | SMA - Female |
| # Outline Drawing | # RD43034100C |

* All Performance Above Are Without Radome.

Dimensions Do Not Include the Connector

| 3. ENVIRONMENTAL |  |  |  |  |
| --- | --- | --- | --- | --- |
| TEST | STANDARD | DURATION | TEMPERTURE | NOTES |
| Low Temperature | IEC 68-2-1 | 72 h | -55°C | - |
| High Temperature | IEC 68-2-2 | 72 h | +71°C | - |
| Temp. Cycling | IEC 68-2-14 | 1 h | -45°C +70°C | 3 Cycles |
| Thermal Shock Nono-Operating |  |  | -30°C to +70°C | Ramp 30°C/min |
| Humidity | ETSI EN300-2-4 T4.1E | 144 h | - | 95% |
| Flammability | UL 94 | - | - | Class HB |
| Salt Spray | IEC 68-2-11 Ka | 500 h | - | - |
| Quasi Random Vibration |  |  |  | 20g rms for 4 hours |
| Vehicle Vibration Operating | 1g rms, 10-500 Hz, in 3 axis |  |  | 6 hours total, 2 hr in each axis. Accelerated wear – an additional 50hrs in worst case axis. |
| Mechanical Shock Operating | 10g, 11 msec, half sine pulse |  |  |  |