Specification for MT – 262054/SLH

LHCP Reader Antenna

902 - 928 MHz, 3dBic Preliminary

THIS DOCUMENT AND THE INFORMATION CONTAINED IN IT ARE PROPRIETARY AND CONFIDENTIAL TO MTI.  NO PERSON IS ALLOWED TO COPY REPRINT REPRODUCE OR PUBLISH ANY PART OF THIS DOCUMENT NOR DISCLOSE ITS CONTENTS TO OTHERS NOR MAKE ANY USE OF IT NOR ALLOW OR ASSIST OTHERS TO MAKE ANY USE OF IT, UNLESS BY THE PRIOR WRITTEN EXPRESS AUTHORIZATION OF MTI AND THEN ONLY TO THE EXTENT AUTHORIZED.

# (C) COPYRIGHT – MTI WIRELESS EDGE LTD.

| REVISION RECORD |  |  |  |  |
| --- | --- | --- | --- | --- |
| DATE | APVD | DESCRIPTION | ECO | REV |
| 11.5.14 | YAEL H. | UPDATE SPEC | -- | 01 |
| 4.8.14 | YAEL H. | UPDATE SPEC | -- | 02 |

| 56 | 55 | 54 | 53 | 52 | 51 | 50 | 49 | 48 | 47 | 46 | 45 | 44 | 43 | SHEET |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | REV |
| 42 | 41 | 40 | 39 | 38 | 37 | 36 | 35 | 34 | 33 | 32 | 31 | 30 | 29 | SHEET |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | REV |
| 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | SHEET |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | REV |
| 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | SHEET |
|  |  |  |  |  |  |  |  |  |  |  | 02 | 02 | 02 | REV |

| # PRODUCT STATUS: |  |  |  |
| --- | --- | --- | --- |
| # PRODUCTION | # PREPRODUCTION | # IN DEVELOPMENT | # PROPOSAL |

| CONTRACT NO |  |  |  |
| --- | --- | --- | --- |
| DATE | SIGN | NAME | ACTION |
| 27.10.13 | YAEL | YAEL HRASH | PREP |
|  |  |  | CHKD |
|  |  |  | CHKD |
|  |  |  | CHKD |
| 27.10.13 | I.S | ISRAEL SARAF | DSGN |
| 27.10.13 | M.R | MOTI ROSTMI | PROJ |
|  |  |  | MFG |
|  |  |  | QA |
|  |  |  | MARK. |

##### SPECIFICATION

Flat Panel Reader Antenna

| #### MTI PART NUMBER |  | ## MT – 262054/SLH |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Regulatory Compliance |  | RoHS , CE 0682 |  |  |  |
| 1. ELECTRICAL |  |  |  |  |  |
| # Frequency Range |  | 902 - 928 MHz |  |  |  |
| Gain |  | 3.0 ± 0.5 dBic |  |  |  |
| VSWR |  | 1.3 :1 |  |  |  |
| 3 dB Beam width Az El |  | 75° (typ) 75° (typ) |  |  |  |
| Polarization |  | LHCP |  |  |  |
| Axial Ratio @ Bore sight |  | 3 dB (typ) , 6 dB (max) |  |  |  |
| F / B Ratio |  | -5 dB (typ) |  |  |  |
| Input Impedance |  | 50 ohm |  |  |  |
| Input Power |  | 6W (max) |  |  |  |
| 2. MECHANICAL |  |  |  |  |  |
| Dimensions(LxWxD) |  | 95 x 95 x 11.0 mm (not include connector ) |  |  |  |
| Weight |  | 100gr |  |  |  |
| Connector |  | SMA - Female |  |  |  |
| # Outline Drawing |  | # RD43796500C |  |  |  |
| 3. ENVIRONMENTAL |  |  |  |  |  |
| TEST | STANDARD |  | DURATION | TEMPERTURE | NOTES |
| low temperature | IEC 68-2-1 |  | 72 h | -55°C | - |
| high temperature | IEC 68-2-2 |  | 72 h | +71°C | - |
| temp. cycling | IEC 68-2-14 |  | 1 h | -45°C +70°C | 3 Cycles |